YO 6.5 YAGI OPTIMIZER

Copyright 1995 by Brian Beezley, K6STI

All Rights Reserved

YO 6.5 Yagi Optimizer..........................1

Running YO.....................................1

F/B and F/R....................................3

Frequency Menu.................................3

Height Menu....................................4

Elements Menu..................................4

Element Tapering...............................5

Element-Mounting Brackets......................7

Boom Compensation..............................8

Conductivity Losses............................9

Frequency Scaling..............................9

Optimization Tradeoffs........................10

Good-Enough Thresholds........................11

Optimization Method...........................12

Undoing Changes...............................13

Matching Networks.............................14

Dual Driven Elements..........................16

Output Files..................................17

Notepad.......................................18

Options Menu..................................18

Gain Figure-of-Merit..........................19

Element-Current Profile.......................20

Frequency Graphs..............................21

Pattern Plots.................................21

Arrays of Yagis...............................23

Algorithm Limitations.........................23

Changing Screen Colors........................24

Printing The Screen...........................25

Graphics Image Files..........................25

Mouse.........................................26

SET Commands..................................26

Index.........................................33

---- YO 6.5 YAGI OPTIMIZER -------------------------------------

YO.EXE analyzes and optimizes Yagi-Uda antenna designs

on your PC.  YO runs hundreds of times faster than NEC- or

MININEC-based antenna-modeling programs.  YO's mathematical

model is more complex than the W2PV Yagi model, simpler than the

MININEC model, and more accurate than either.  YO accounts for

conductor losses; it models Yagis with dual driven-elements; it

models antennas in free space or over an infinite, perfectly

conducting groundplane; it can model two identical Yagis stacked

in the H-plane in free space.  YO calculates mutual-impedance

interactions in all of these cases.  YO also can ignore mutual

impedances and synthesize the pattern and gain of a large,

rectangular array of free-space Yagis from the pattern of a

single antenna.

YO includes an automatic optimizer to simultaneously

maximize forward gain and input impedance, improve the radiation

pattern, and minimize SWR across a frequency band.  This

iterative procedure automatically adjusts all element lengths

and spacings to optimize performance criteria and parameter

tradeoffs you specify.

YO is calibrated to NEC, the reference-accuracy

Numerical Electromagnetics Code from the Lawrence Livermore

National Laboratory.  YO and NEC typically are within 0.05 dB

for forward gain, a dB or two for F/B, and a couple ohms for

input impedance.  Radiation patterns calculated by YO and NEC

are virtually identical for most designs.  For comparison, YO

also provides a MININEC calibration.

---- RUNNING YO ------------------------------------------------

YO stores information for Yagi designs in individual

files.  Yagi files contain element dimensions, analysis

frequencies, and other information.  You specify the name of a

Yagi file when you start YO.  After YO loads the file, you can

examine or modify the design.  You can save the current design

in a file at any time.  Yagi files use the extension .YAG.

If you know which Yagi file you want, give the filename

on the YO command line, for example, YO PV4 (the extension isn't

needed).  Otherwise, type YO and Yagi files in the current

directory will be listed.  To list Yagi files in a different

directory, specify the directory name on the command line.

Select a file by moving the lightbar with the arrow

keys, PgUp, PgDn, Home, or End.  Press Enter to select the

highlighted file.  Alternatively, you can enter a filename by

typing it.  As you type, the lightbar moves to the first

filename that matches the characters entered.  Press Enter

whenever the desired file is highlighted.  The last alternative

is simply to click on a filename with your mouse.

1

Select "Other" to enter a file or directory name not

listed.  If you enter a directory containing Yagi files, YO

lists them and you can select one.  See the SET Commands section

for information on organizing Yagi files into subdirectories.

Instead of analyzing a Yagi file, you can display a plot

file generated previously.  To do this, start YO by typing YO

PLOT.  YO will list plot files in the current directory and you

can select one.  Alternatively, you can specify the name of a

plot file on the command line, for example, YO PLOT PV4.  This

displays the plot immediately.

Press Esc to hide the Main Menu to view the whole

screen.  Press Esc, Enter, or the spacebar to bring it back.

Press Esc to return to the Main Menu from a submenu.  You also

can return by pressing the command key a second time (but not

from Save).  This feature is handy when making a quick check.

You can terminate data entry and return to the Main Menu

with one keystroke by pressing Esc instead of Enter.  If you do

this, whenever you return to the submenu the cursor will be

positioned at the item last entered.  This makes it easy to

change a parameter repeatedly to experiment with its effect.

You can exit most submenus (not Save or Notes) by

pressing a valid command key.  YO will execute the command

immediately, bypassing the Main Menu.  This is a quick way to go

from one submenu to another.  Whenever you press a command key

during data entry, YO accepts the data and then executes the new

command.

Use an arrow key with numerical input to enter data and

simultaneously move the cursor up, down, right, or left to

another item.  Use the spacebar to enter data without advancing

to another item.  This is particularly useful with the Match

Menu and Bracket Menu to change a parameter value repeatedly to

experiment with its effect.

To reduce screen clutter, YO does not label the numbers

within Yagi patterns.  They have the following meaning:

Frequency

Forward Gain

Front-to-Rear Ratio

Input Impedance

Standing-Wave Ratio

Elevation Angle or Gain FOM

YO displays elevation angle only for Yagis over ground

and gain figure-of-merit only for single-Yagi, free-space

models.  YO defaults to a generalized definition of front-to-

back ratio described in the next section.

The notation 12.7-j15.4 means a resistance of 12.7 ohms

in series with a reactance of -15.4 ohms.  Z stands for

impedance.  The lambda symbol stands for wavelengths (when

2

you're not using YO fonts, YO substitutes the letter w).  The

path and name of the current Yagi file appear in the lower-right

screen corner.  YO increments the iteration count in the lower-

left corner each time it recalculates element currents.

---- F/B AND F/R -----------------------------------------------

YO uses a generalized notion of standard front-to-back

power ratio to characterize pattern quality.  Conventional F/B

is the ratio of forward power (at 0 degrees) to power radiated

in the opposite direction (at 180 degrees).  YO's generalized

F/B is the ratio of forward power to power radiated within a

specified region to the rear of the antenna.  This is called

front-to-rear ratio (F/R).  Yagi designs maximizing conventional

F/B often have large backlobes at angles other than 180 degrees.

Much better patterns result when you optimize a Yagi for F/R.

The F/R region begins at 180 degrees and extends forward to an

angle you specify (90 degrees by default).  You can define the

forward limit in 5-degree steps and you can set it as far

forward as 5 degrees.  This wide range allows you to control

sidelobes as well as backlobes.

YO provides two methods for characterizing power within

the F/R region.  The first method simply averages power.  When

you optimize F/R using this method, you minimize the total power

within the F/R region.  This is particularly useful with EME

arrays to minimize total earth noise on receive.  The second

method uses an L4 norm.  This mathematical function is similar

to that used to compute RMS power, but the largest amplitudes

are weighted much more heavily.  F/R computed using the L4 norm

is approximately the ratio of forward power to the power of the

peak backlobe within the F/R region.  When you optimize F/R

using peak power, you minimize backlobe amplitudes while

maintaining roughly equal backlobes.  This method is

particularly useful with HF Yagis since it minimizes worst-case

interference from signals arriving at random rear angles.  YO

samples the radiation pattern in 5-degree steps to compute F/R.

You can change the F/R region with the Tradeoffs Menu.

Set it for 180 to 180 degrees (a single point) to obtain

conventional F/B.  Select peak or average backlobe power from

the Options Menu.  The F/R value depends on the analysis plane,

on the extent of the F/R region in that plane, and on the power-

computation method.

For H-plane optimization of antennas over ground, the

F/R region begins at the rear horizon and extends upward to the

selected elevation angle.  This permits you to minimize rear

radiation over a range of elevation angles.

---- FREQUENCY MENU --------------------------------------------

YO models Yagi designs at a spot frequency or over a

frequency band.  Specify a single analysis frequency for spot-

3

frequency designs, and low, middle, and high frequencies for

designs covering a band.

You can add, change, or delete analysis frequencies with

the Frequency Menu.  You must use one or three frequencies--YO

doesn't allow two.  To change to a spot-frequency design, enter

zero for the low or high frequency.

Use the Options Menu to select optimization at all three

frequencies or at the middle frequency only.  It takes three

times as long to optimize at all frequencies.  When optimizing

at one frequency, it takes additional time to compute and

display the response at the other two frequencies.  For maximum

speed, use the fewest number of frequencies.

YO can model Yagis to 10 GHz.  Special tweezers are

available for those of you working with Yagis near the upper end

of this range.

---- HEIGHT MENU -----------------------------------------------

Use the Height Menu to specify the height above ground

for a single Yagi or the free-space, H-plane stacking distance

for two.  YO orients a Yagi for horizontal polarization when

modeling over ground.  Set height to zero to model Yagis in free

space.  Set stacking distance to zero to model a single Yagi.

YO computes forward gain, F/R, and the azimuth pattern at the

specified elevation angle.

YO takes into account mutual impedances between Yagi

elements and their ground images and between stacked Yagis.

This lets you see the effect of ground or antenna proximity on

azimuth patterns and input impedance.  However, YO models

perfectly conducting ground, not real earth with finite

conductivity and dielectric constant.  Yagis over real earth may

have several dB less gain overhead and up to 1 dB less gain near

the horizon than the figures YO computes.  Elevation-pattern

nulls may be less deep.  Nevertheless, you can use YO to make

accurate relative comparisons among Yagi designs over ground.

Use NEC/Yagis to accurately model a Yagi over real earth.

---- ELEMENTS MENU ---------------------------------------------

Use the Elements Menu to change element positions and

lengths.  YO displays element half-length, the distance from the

center of the boom to element tip.  For tapered elements YO

displays the lengths of electrically equivalent untapered

elements.  These are not physical lengths and should not be used

for antenna construction.  Use the Taper Schedule Menu to

inspect and change physical lengths of tapered elements.

YO can model Yagis with up to 50 elements.  Use the Ins

and Del keys to add or delete elements.  Press Del to delete the

element at the cursor.  Press Ins to add an element below the

4

cursor.  When you add or delete an inner element, YO respaces

elements to avoid bunching or gaps.  When you add or delete an

end element, YO does not respace and the boomlength changes.

Use the X key to fix or free element position or length

during automatic optimization.  The * key fixes or frees the

positions or lengths of all elements.  Use * then X to free just

a few elements.  YO highlights fixed element dimensions by

color.

Use the D key to toggle between single and dual driven

elements.  Whenever you switch to dual, consider enabling both

dual-drive options in the Match Menu to help optimize what was

previously a single-driven design.  Any toggle frees all element

dimensions (except the length of a single driven element).

Use the ~ key to round element positions or lengths.

Set the cursor to the column of numbers you want to round.  This

is a good way to test design sensitivity to dimension

tolerances.  You can also use rounding to generate final

dimensions for construction.  Use Undo to undo rounding

experiments.  (Since YO rounds the tip sections of tapered

elements, it's normal for untapered-equivalent lengths not to be

multiples of the rounding value.)

Use the = key to equalize element spacing or lengths.

Set the cursor to the column you wish equalized.  All elements

below the cursor are affected.  You can equalize element spacing

to provide a good starting point for automatic optimization.

You'll probably not want to equalize element lengths unless a

design has become hopelessly screwed up.  Use Undo to recover

from equalization experiments.

Changing the position of the last director has a special

effect:  All element positions after the last fixed-position

element change to expand or shrink the design.  The whole beam

scales if no element positions are fixed.  This feature is very

handy when experimenting with boom length.

YO expects every Yagi to have a reflector element.  You

can model a two-element Yagi with driven element and director by

placing a dummy reflector a great distance away from the two

active elements.  This arrangement will satisfy YO while leaving

the response of the two-element array unaffected.

Before accessing the Elements Menu, you can select an

element from the Main Menu with the left/right arrow keys.  The

selected element blinks in the Yagi sketch.  When you bring up

the Elements Menu, the cursor will be positioned at the selected

element.  This feature makes it unnecessary to count elements.

---- ELEMENT TAPERING ------------------------------------------

Fast Yagi-modeling algorithms represent each element as

a thin cylinder of constant diameter.  A tapering algorithm is

5

necessary to convert elements of tapered telescoping tubing to

untapered cylindrical equivalents.  For practical purposes,

tapered and untapered elements are electrically equivalent when

their self-impedances are equal.  Tapered elements are always

physically longer than their untapered equivalents.  Tapering

must be accurately modeled to realistically characterize HF

Yagis.

YO can use one of three tapering algorithms.  The first

is the method developed by James Lawson, W2PV, from the ARRL

book "Yagi Antenna Design."  While this algorithm has been

widely used for a number of years, it doesn't fare too well when

combined with an accurate Yagi analysis algorithm.  The W2PV

method works reasonably well with MININEC, but only because

frequency biases of the two algorithms tend to cancel.  A

MININEC study of a number of tapered dipoles and their W2PV

untapered equivalents showed that element self-impedances were

not very close.  In addition, self-impedance values diverged

rapidly with frequency excursion from the central design

frequency.

YO's second tapering algorithm is designed to correct

these shortcomings.  This algorithm modifies the W2PV method to

yield untapered-equivalents with self-impedances much closer to

those of the tapered originals.  It also better predicts the

real-world performance of Yagis with heavily tapered elements.

In addition to providing better equivalent lengths, the

modified-W2PV algorithm also calculates untapered-equivalent

diameters more accurately.  It calculates diameters in such a

way that the self-impedance of untapered and tapered equivalents

match over a wide frequency range, not just at a single

frequency.  In essence, the taper conversion doesn't alter

element Q.  In contrast, standard W2PV implementations set the

untapered-equivalent diameter to 7/8", to the mid-element taper

diameter, or to the average diameter of the tubing sections.

Such equivalent diameters are approximately correct for many

taper schedules in common use.  However, to accurately model

Yagi performance over a frequency band, either the equivalent

diameters must be accurately represented or the taper conversion

must be recalculated at each analysis frequency.  Self-impedance

errors away from the central design frequency can cause modeling

inaccuracies at the band edges.

YO's third tapering algorithm is by Dave Leeson, W6QHS,

from the ARRL book "Physical Design of Yagi Antennas."  This

algorithm gives results similar to those of the modified-W2PV

method.  The W6QHS algorithm has the advantage of being based on

fundamental principles rather than on an empirical data fit.  It

has the disadvantage of not having been fine-tuned to the biases

of the YO analysis kernel.

Use the Options Menu to select the tapering algorithm.

The untapered-equivalent diameter used for the original W2PV

method is the average diameter of the taper sections, where the

diameter of each section is weighted by section length.  This

6

avoids problems introduced by very short taper sections

representing element-mounting brackets.

YO can model Yagis with up to 20 taper sections per

half-element.  Use the Taper Schedule Menu to change the

diameter or length of any section.  For any length change except

to the tip section itself, YO readjusts tip length to maintain

an electrically equivalent design.  Use the Ins and Del keys to

add or delete taper sections.  Use the = key to equalize taper

lengths below the cursor in a column.  This is handy when making

wholesale changes to a taper schedule.  Use a taper length of

zero for elements that don't use a particular tubing diameter.

---- ELEMENT-MOUNTING BRACKETS ---------------------------------

A conductive element-to-boom mounting bracket increases

element effective diameter at the bracket.  The amount of

increase depends on the size and shape of the bracket.  YO uses

the equations of D. Jaggard to model flat, rectangular mounting

plates, Hy-Gain element clamps, L-angle brackets, and U-channel

brackets.  For details of the method, see the ARRL book

"Physical Design of Yagi Antennas" by Dave Leeson, W6QHS.

Use the Bracket Menu to define or modify a mounting

bracket.  Select bracket type with the tab key.  After you enter

bracket dimensions, YO calculates the length and diameter of an

equivalent taper section.  The length of the section is the

half-length of the bracket.  Its diameter is electrically

equivalent to the element and bracket combined.

When you return to the Main Menu, YO updates all

elements of the current design with the new taper section.

Whenever the length of the first taper section is less than 5%

of the element half-length, YO assumes that the section already

represents a mounting bracket.  YO will modify the section

instead of adding a new one.

If you enter zero for a bracket dimension so that YO

lists "None" for the equivalent taper, when you return to the

Main Menu YO removes the first taper section if it represents a

mounting bracket (length less than 5% of the element half-

length).

If the Yagi uses mounting brackets of more than one

size, calculate equivalent taper sections with the Bracket Menu

and add them to individual elements with the Taper Schedule

Menu.  (YO uses reflector tapering to calculate the numbers

displayed in the Bracket Menu.  You may need to temporarily

modify the taper schedule of this element to do calculations for

other elements.)  Use a taper length of zero for elements that

don't use a particular taper diameter.

To avoid disrupting a complex taper schedule entered by

hand, YO will not update the taper schedule when you leave the

Bracket Menu if the length of the first taper section is zero

7

for any element.  It also won't update if some elements have

mounting brackets and others do not.

The length of a mounting bracket is its dimension along

the element.  Enter the total length (both sides of the boom).

The width of a bracket is its dimension perpendicular to the

element.  The thickness of a Hy-Gain clamp is the total

thickness of the flat part.  Height has no meaning for flat

plates and YO automatically computes this number for Hy-Gain

clamps.  For angle or channel brackets, height refers to the

length of the vertical side(s).  YO assumes that the element

touches two surfaces of an angle bracket and one surface of a

channel bracket.

It's important to take element mounting into account

when modeling Yagis.  Large mounting brackets may significantly

alter antenna characteristics.  In extreme cases they can move a

desired response completely outside a band.  Small brackets can

upset a carefully optimized pattern at a spot frequency.

However, mounting methods that use compact hardware no larger

than the element diameter generally require no correction.

---- BOOM COMPENSATION -----------------------------------------

When an element is close to or passes through a

conductive boom, its electrical length changes.  YO does not

model this boom effect, but you can use the information in this

section to compensate manually.  All correction values apply to

element half-lengths.  Apply the corrections by increasing

calculated element lengths when constructing a Yagi.

Conversely, to model boom effects you must shorten measured

element lengths.

For elements mounted on flat plates in contact with a

boom, W2PV found that element half-lengths should be corrected

by 3% of the boom diameter.  He states that this small effect

diminishes rapidly as the element is spaced away from the boom,

even by a small distance.

For noninsulated through-the-boom mounting, measurements

by Guenter Hoch, DL6WU, were curve-fit by Ian White, G3SEK, to

yield the following boom-correction formula:

C = (12.5975 - 114.5 * B) * B * B

C is the element half-length correction and B is boom

diameter, both in wavelengths.  * means multiply.  For example,

a 0.01-wavelength-diameter boom requires element half-lengths to

be corrected by 0.00115 wavelength.  The experimental data

underlying this formula came from booms with diameters smaller

than 0.055 wavelength; the formula isn't valid for larger booms.

G3SEK says that the correction required for insulated through-

the-boom mounting is close to 50% of C.

8

---- CONDUCTIVITY LOSSES ---------------------------------------

YO accounts for ohmic losses in Yagis due to imperfect

element conductivity and skin effect.  These losses usually are

small for tubing elements at HF, but they can be significant for

rod elements at VHF/UHF.  When computing losses, YO takes into

account the element-current distribution, the position, length,

and diameter of taper sections, and skin-effect depth, which

depends on frequency, material resistivity, and material

permeability.

Press C in the Elements Menu to select conductor

material.  You can pick one of the following:

Silver           Pure silver

Copper           Int'l Annealed Copper Std

Alum             Pure aluminum

6063             6063-T832 aluminum alloy

6061             6061-T6 aluminum alloy

Brass            Yellow brass (35% zinc)

Bronze           Phosphor bronze (5% tin)

SSteel           Stainless steel type 302

Note that 6061-T6 material has a volume resistivity 23%

higher than that of 6063-T832 alloy and 51% higher than that of

pure aluminum.

You can enter resistivity values directly to model

materials not listed.  Enter a value of zero to model perfect-

conductivity elements.  When you do this, forward gain increases

by an amount equal to total conductor loss.  You can enter

permeability values greater than 1 to model ferromagnetic

material like iron or steel.

---- FREQUENCY SCALING -----------------------------------------

YO can scale any design to a new frequency while

maintaining essentially the same performance characteristics.

This makes it easy to replicate a Yagi design on another band

without having to design a new antenna from scratch.  The scaled

design can use a completely different taper schedule.

Although scaling can virtually duplicate Yagi

performance at a new spot frequency, antenna response won't be

exactly the same across a new frequency band unless untapered-

equivalent element diameters scale in proportion with frequency.

This is unlikely since practical mechanical considerations or

tubing material at hand normally dictate element diameters.

Moreover, the new frequency band is unlikely to have the same

percentage bandwidth as the old.  Therefore, after scaling a

design, reset the low and high frequencies to the new band edges

and check performance across the band.  You can do a quick,

touch-up optimization if necessary.

9

Frequency scaling in YO does not rely on a simple

polynomial approximation for element self-impedance as does the

W2PV scaling technique, nor does it attempt to match element

reactances.  Instead, YO iteratively computes element self-

impedance with the method it uses for normal analysis while

adjusting tip length.  YO stops adjustment when the phase angle

of the complex self-impedance has converged to that of the

original element.

To scale a design, use the Taper Schedule Menu to enter

new taper diameters and lengths.  (Don't bother to set tip

lengths since YO will adjust these during scaling.)  Then press

C and enter the new design frequency.  When you press Esc, YO

adjusts element tips to provide a design at the new frequency

electrically equivalent to the design current when you brought

up the Taper Schedule Menu.

Instead of scaling a design to a new frequency, you may

just want to change taper diameters while maintaining the same

electrical design.  To do this enter the new diameters, press C,

and then press Esc without entering a new frequency.  This trick

is useful when changing from standard U.S. to standard metric

tubing diameters.

---- OPTIMIZATION TRADEOFFS ------------------------------------

Maximizing Yagi forward gain produces the strongest

transmit signal in the forward direction.  On receive, it yields

the best signal-to-noise ratio for omnidirectional noise.

However, it also results in a narrow main beam, a large

backlobe, multiple sidelobes, low input impedance, and small SWR

bandwidth.  Yagis optimized for good patterns sacrifice gain,

while those designed for wide SWR bandwidth give up gain and

pattern quality.

Because of the inherent tradeoffs in Yagi design, YO

permits you to optimize a combination of forward gain, F/R, SWR,

and input impedance.  You select the combination of parameters

and their relative importance with the Tradeoffs Menu.

YO expresses parameter tradeoffs in percent to indicate

relative importance.  For example, to maximize forward gain

regardless of the other parameters, set the gain tradeoff to

100%.  To maximize gain but also take F/R into account, set the

F/R tradeoff to some nonzero value, say 50%.  This tells YO to

weigh both forward gain and F/R when optimizing, and in this

case, to give them roughly equal importance.  YO does this by

forming a weighted sum of the forward gain and F/R values and

then maximizing the sum.

If you allow optimization to run to completion, the

final design will be optimal in the following sense:  Any small

design change that yields a gain improvement of X dB will

necessarily reduce F/R by at least Y dB, where Y is related to X

by the ratio of the optimization summing weights.  In other

10

words, there will not be any free gain or free F/R to be had,

and to improve one parameter, the other must be sacrificed by a

factor proportional to its relative importance.

YO provides two additional tradeoffs for use in

combination with the gain and F/R tradeoffs.  The SWR tradeoff

minimizes SWR, while the impedance tradeoff raises input

resistance.  YO evaluates SWR using an L4 norm to better

minimize peak SWR across a band.  You can use the impedance

tradeoff to prevent the very low impedances characteristic of

maximum-gain designs.

Often you can significantly reduce SWR with very little

loss in forward gain or F/R.  After you've optimized a design

without regard for SWR, try introducing a small SWR tradeoff,

even if the SWR is already acceptable.  Similarly, you can often

raise input resistance without significant loss of performance

by experimenting with a small impedance tradeoff.

Using equal values for the gain, F/R, and SWR tradeoffs

usually yields a reasonable, balanced Yagi design.  (The

impedance tradeoff normally isn't needed for three-frequency

designs, while the SWR tradeoff usually is superfluous for

single-frequency designs).  The default tradeoff values form a

convenient starting point for custom optimization.  Vary the

tradeoff percentages according to the importance you place on

each design parameter.  The values themselves are not important;

it is their effect that counts.  For example, to emphasize F/R,

increase the F/R tradeoff value.  If YO sacrifices too much gain

for SWR, raise the gain tradeoff or lower the SWR tradeoff.  By

adjusting the tradeoff percentages, you can home-in on a design

that meets your requirements.  The design will be optimal in the

sense that an optimal tradeoff will have been made among the

performance parameters according to the importance you assigned

to each.

During optimization, YO does not allow elements to

migrate closer than 0.05 wavelength.  To change this limit, see

the SET Commands section.

---- GOOD-ENOUGH THRESHOLDS ------------------------------------

Sometimes a parameter of interest will greatly exceed

your design goal at one or two frequencies.  For example, you

might obtain more than 35 dB F/R at the high end of a band even

though all you really need is 20 dB.  Whenever a parameter

exceeds a design goal, invariably another parameter is

sacrificed to provide the excess performance.  You can reduce

the F/R tradeoff to bring the optimizer's goals more in line

with your requirements.  But since the tradeoffs affect all

frequencies, F/R may then drop unacceptably low at the worst

frequency.

Good-enough thresholds can help in this situation.

These numbers provide limits beyond which YO does not regard an

11

increase in performance as an improvement.  For example, if you

set F/R OK > 20 dB in the Tradeoffs Menu, whenever F/R exceeds

20 dB, the optimizer will not regard it as an improvement.  It

has no incentive to improve F/R at a frequency as long as it

stays above 20 dB.  This frees the optimizer to change the

design in ways that maximize your other objectives, like forward

gain.

It's probably best to disable the good-enough thresholds

(by setting them to extreme values) when you first begin a new

design.  YO defaults to this condition.  Some designs sacrifice

little in one parameter to achieve very high performance in

another.  If you habitually set good-enough thresholds near

tolerable levels you'll never uncover these remarkable designs.

Use the tradeoff percentages to proportion optimizer

driving power according to your design priorities.  Use the

good-enough thresholds to dynamically redirect that power once

an objective has been met.  Unless you use a great number of

elements, Yagi characteristics correlate across a frequency

range.  This means that good-enough thresholds cannot completely

flatten antenna response across a band.  But often they can

help.

---- OPTIMIZATION METHOD ---------------------------------------

YO uses a conjugate-gradient optimizer.  This method is

more effective than the steepest-descent technique used by

previous YO versions.  The optimizer works as follows:

First, YO changes each element length and position by a

very small amount (with all other dimensions unchanged) to

calculate the sensitivity of the objective to each variable.

The objective is the weighted combination of gain, F/R, SWR, and

impedance that you've defined with the tradeoffs and good-enough

thresholds.  The set of element sensitivities is called a

gradient.  The gradient is an n-dimensional vector, where n is

the number of element lengths and positions you're varying.  The

gradient vector points in the general direction that maximizes

the objective.  After YO computes the gradient, it recalculates

Yagi response at points along the gradient vector seeking the

maximum value of the objective function.  When it's found, YO

calculates a new gradient and forms a new search direction.  The

new direction utilizes information from old in a way that

minimizes the number of iterations required.  This process

continues until the optimizer can no longer improve the

objective function.

This optimization technique is quite stable and

efficient, locating optimal dimensions for an antenna without

wild-goose chases in a reasonable number of iterations.

However, the method does not guarantee that the global optimum

is found (nor does any other optimization technique, except for

an impractical, exhaustive search over all possible Yagi

geometries).  The global optimum is the very best set of antenna

12

dimensions possible, where "best" is defined by your tradeoffs

and good-enough thresholds.  The global optimum is missed when

the algorithm converges to a local optimum, where small movement

in any direction produces inferior results.

Optimization using perturbation techniques is inherently

nearsighted.  If you're maximizing forward gain, for example,

and there's a small bump in the gain curve for a certain set of

element lengths and positions, the design may wind up on top of

the bump because the algorithm cannot see beyond the immediate

vicinity to the highest point of the overall curve.  If you

imagine trying to climb to the very top of a mountain range with

multiple peaks in dense fog, you can visualize the problem.

Fortunately, you won't often be troubled by local optima

with Yagis.  They usually occur only when a design undergoes

radical change.  If you believe that your design has converged

to a local optimum, you can modify antenna dimensions manually

and continue automatic optimization in a different region of the

performance space.

To avoid terminating optimization prematurely, YO

continues to iterate even when very small performance

improvements are being achieved.  Sometimes this slowly inches a

design into a new region of the performance space where

significant improvement becomes possible.  However, whenever you

think further improvement is unlikely, you can interrupt the

optimizer by pressing a key.  Press Esc, Enter, or the spacebar

to interrupt the optimizer with no further action.  If you press

a command key, the command will be executed immediately.  If you

start the optimizer with Alt-Z and let it run to completion, it

will beep twice when it's done.

---- UNDOING CHANGES -------------------------------------------

YO automatically saves the current Yagi design on a

last-in, first-out stack whenever the design changes.  The stack

maintains a history of design changes.  You can recall a

previous design with the Undo command.  Each press of U moves

back one design.  Press Alt-U to move all the way back to the

first design on the stack.  Use the Redo command to undo an

undo.  Each press of R moves forward one design, while Alt-R

moves all the way forward to the latest design.

Undo and Redo make it unnecessary to periodically save

intermediate designs with the Save command.  They make it easy

to undo manual element changes since you don't have to remember

the old dimensions.  Undo and Redo provide a great way to

compare results before and after optimization, particularly when

you use graph overlays or graph comparison.

Undo and Redo have several limitations.  Conductor

material, matching networks, and mounting-bracket dimensions

aren't saved on the stack (but taper sections representing

brackets are).  Optimized designs are saved, but intermediate

13

designs generated during optimization aren't.  Finally, if you

change a recalled design, the design history from that point

forward is lost.

---- MATCHING NETWORKS -----------------------------------------

The Match Menu lets you design a variety of matching

networks.  Use the tab key to select one of the following

network types:

1.  Perfect Match

The perfect match is an idealized matching network.

It's always perfectly matched at one frequency (called the match

frequency).  The match frequency that best minimizes SWR across

a band isn't necessarily the center frequency, so YO adjusts the

match frequency during optimization whenever the SWR tradeoff is

nonzero.  You can set and fix the match frequency to favor a

certain portion of a band.  This disables automatic adjustment.

To reenable it, set the match frequency to zero.

Although the optimizer can minimize SWR while using

another matching network, it won't adjust network parameters

automatically.  The perfect match is convenient for optimization

because it's always perfectly matched at some frequency and YO

will automatically adjust that frequency to minimize SWR.

The perfect match is implemented like this:  YO uses the

complex conjugate of antenna input impedance at the match

frequency as the SWR reference impedance at all frequencies.

This effectively models a perfectly broadband matching network.

Therefore, SWR variation using the perfect matching network

reveals the inherent impedance-bandwidth properties of the

antenna.

Setting the match frequency to the center frequency

increases speed about 25% when optimizing at three frequencies.

This lets YO obtain results with three Yagi evaluations rather

than four.  Using an SWR tradeoff of zero has almost the same

effect.  In this case YO must calculate Yagi response at the

match frequency only for infrequent screen updates.

2.  Hairpin Match

The hairpin match is an L-network using distributed

reactances.  The driven element is shortened from its resonant

length to raise the equivalent parallel input resistance to that

of the feedline.  The hairpin acts as a shunt inductance to

cancel the capacitive reactance of the shortened driven element.

Hairpin matches are commonly fed with coax through a balun.  YO

models the hairpin as a transmission line of parallel rods.

14

You specify rod diameter, rod length, and center-to-

center rod spacing.  You can also specify balun-lead length.

Enter the length of one lead from the axis of the balun to the

element-attachment point.  YO assumes the leads use #12 wire.

At HF, YO models the leads as fanning out from a spacing of 1.5"

at the balun to the hairpin-rod spacing at the element.  For

frequencies above 30 MHz, YO assumes the leads fan out from

0.15".  This lead spacing is typical of 3/8"-diameter coax

(presumably loaded with a few ferrite beads).

You can specify shunt capacitance between driven-element

halves.  This is particularly useful with Hy-Gain element

clamps.  Use 26.5 pF for 20-meter clamps and 9.5 pF for 15- and

10-meter clamps.

Hy-Gain calls its version of the hairpin match a beta

match.  It uses two rods that straddle the boom and a shorting

strap that connects all three together.  The presence of the

boom has a negligible effect on hairpin inductance, so a beta

match is electrically equivalent to a conventional hairpin

match.

NOTE:  Driven-element current nearly vanishes between

the hairpin-rod attachment points.  Therefore, measure driven-

element half-length from the attachment point rather than the

center of the boom.  Except for shunt capacitance, it's not

necessary to model the driven-element mounting bracket.

3.  Gamma Match

YO uses equations developed by Harold Tolles, W7ITB, to

model gamma matches.  You specify gamma-rod diameter, rod

length, center-to-center spacing from the driven element, and

series capacitance.  To model lead inductance, specify the lead

length from the gamma rod to the coax connector.  YO assumes the

lead uses #12 wire.  When the lead runs parallel to the driven

element, include its length as part of the gamma rod rather than

specifying it separately.

4.  T Match

A T match is a balanced gamma match.  Rod length and

capacitance values apply for each side of the driven element.

Some T matches are fed with 50-ohm coax through a 4:1 balun.

Enter a feed impedance of 200 ohms for these systems (which

often have negligible lead length).  You can also feed a T match

with wire leads from a 1:1 balun.  YO makes the same assumptions

about these balun leads as it does for a hairpin match.  YO

assumes the distance between T-match rod ends is half their

distance to the driven element.  To model a T match without

capacitors, enter a high capacitance value.  To model a folded

dipole, extend the T-match rod to the end of the element.

15

Gamma- and T-match rods increase the effective diameter

of the inner part of a driven element.  Because YO does not

model this effect, you may need to compensate by increasing

driven-element length when constructing a Yagi.

YO uses a velocity factor of 0.975 for the transmission

lines formed by gamma- or T-match rod and driven element,

hairpin rods, and balun leads.  For driven-element diameter, YO

uses a weighted logarithmic average of the diameters of the

taper sections spanned by gamma- or T-match rods (diameters are

weighted by taper-section length).  YO draws rod length to scale

on the Yagi sketch, but not rod spacing.

Matching-network performance depends critically on a

number of parameters that are difficult to measure and to model

accurately.  Calculated and actual matches may differ due to

input-impedance modeling error, series- or shunt-capacitance

estimation error, lead-spacing variation, proximity of other

antennas or guy wires, etc.  When constructing a Yagi, always

make the matching network adjustable!

---- DUAL DRIVEN ELEMENTS --------------------------------------

YO can model Yagis that use two driven elements

interconnected by a crossed phasing line.  This type of Yagi was

first popularized by KLM Electronics, Inc.  Dual-driven designs

sometimes are capable of lower SWR across a frequency band than

conventional designs.  You can toggle dual drive by pressing D

in the Elements Menu.

Dual-driven Yagis manufactured by KLM use balanced feed

at the forward driven element.  The nominal feed impedance is

200 ohms and the antennas use a 4:1 balun with 50-ohm coax.

Phasing-line characteristic impedance varies with KLM antenna

model and with line assembly, but is in the range of 200 to 250

ohms for HF Yagis.  YO uses default values of 230 ohms for

phasing-line impedance and 200 ohms for feed impedance.  You can

specify other values with the Tradeoffs Menu.  YO uses a

velocity factor of 0.975 for the phasing line.

SWR characteristics of dual-driven Yagis are quite

sensitive to phasing-line characteristic impedance.  For KLM

construction, the impedance varies with phasing-line tension.

Two insulators determine line spacing at the center crossover

point, but the spacing elsewhere varies with line tension.  This

affects average line impedance.  Measurement of a 14-MHz KLM

Yagi with a time-domain reflectometer and a vector-impedance

meter yielded an average impedance of 215 ohms for a somewhat

slack phasing line and 240 ohms for a taut line.

Because dual-driven Yagis use direct feed, YO replaces

the normal Match Menu with a special menu with two options.  The

first option allows automatic optimization of phasing-line

impedance.  The second option maintains constant spacing between

16

driven elements while allowing their joint position to vary.

This lets you optimize joint driven-element position without

having to reconstruct an existing phasing line.  Unless you fix

them, YO normally adjusts the lengths of both driven elements

during optimization.

A good strategy for optimizing a dual-driven Yagi is to

first optimize the antenna using a nominal phasing-line

impedance of 230 ohms.  Once you have a good design, save it,

and then enable automatic optimization of phasing-line

impedance.  Reoptimize to see if significant improvement is

possible.  Try various starting impedance values.  Only SWR

characteristics are significantly affected by phasing-line

impedance, but dramatic improvement often is possible.

To realize optimum SWR characteristics, you should be

prepared to adjust the phasing-line impedance of a KLM antenna.

Even if the computer model looks good at 230 ohms, your

particular phasing line may exhibit a different impedance.  It's

easy to change line impedance by altering the spacing between

phasing straps in the crossover region.  Simply replace the

original KLM spacers with larger or smaller ones of your own

design.  Experiments have shown that average line impedance can

be raised by 50 ohms by increasing the crossover spacing to a

few inches and lowered by 50 ohms by decreasing spacing to a

fraction of an inch.  You don't need to measure phasing-line

impedance.  Simply adjust the line spacing until SWR

characteristics match those predicted by YO.  (If you're

unwilling to adjust phasing-line impedance, you can reduce SWR

by adjusting driven-element lengths instead.)

When YO generates an AO-format output file for a dual-

driven Yagi, it uses a current source at each driven element

rather than modeling the phasing line with wires.  Note that

source magnitudes and phases will not be strictly valid if you

change the AO analysis frequency.

NOTE:  Driven-element current nearly vanishes between the

transmission-line attachment points.  Therefore, measure driven-

element half-length from the attachment point rather than the

center of the boom.

---- OUTPUT FILES ----------------------------------------------

Use the Save command to save the current Yagi design in

a file.  You can specify a path with the filename.  If you enter

the name of a file that already exists, YO asks permission

before overwriting it.  For fast periodic saves, press Enter to

overwrite the last file saved (YO doesn't ask permission).  When

no file has been saved, YO saves the design in SAVE.YAG.  YO

does not save matching networks or mounting-bracket dimensions.

In case you forget to save a design, YO automatically

saves the current design in the file OUT.YAG whenever you exit

17

the program.  You can recover the design by specifying OUT.YAG

as an input file.

You can print an output file by entering PRN or LPT for

the output filename in the Save command.  This will record the

antenna geometry in a compact, easy-to-read format along with

any design notes.

If you specify the extension .ANT when saving a file, YO

uses the AO file format.  YO normalizes element positions for AO

files so that the center of the boom is at X = 0.  For YO-format

output files, YO normalizes element positions so that the

reflector position is zero.  YO writes output files using the

units specified in the Options Menu.

---- NOTEPAD ---------------------------------------------------

You can enter, edit, and display design notes with the

Notes command.  YO displays the Yagi title at the top of the

notepad and you can edit it, too.  YO saves the title and notes

in all output files.

The notepad editor automatically wraps words within a

paragraph.  A blank line or a line with leading space begins a

new paragraph.  Use the arrow keys, Home, End, PgUp, and PgDn to

move the cursor.  Press Ctrl with the right or left arrows to

move the cursor to the beginning of the next or previous word.

Use Del or Backspace to delete characters and Alt-D to delete a

line.  Press Alt-W to delete a word (when the cursor is between

words, all spaces but one are deleted).  See the SET Commands

section to redefine the keys for word delete and line delete.

Press PrtSc to print the notepad screen.

---- OPTIONS MENU ----------------------------------------------

The Options Menu lets you control some additional

aspects of YO.

The E-plane is the plane containing Yagi elements.

Select this plane to optimize the azimuth pattern of a

horizontally polarized Yagi.  The H-plane is the plane

perpendicular to the elements.  Select this plane to optimize

the azimuth pattern of a vertically polarized Yagi.

You can disallow longer elements during optimization.

This is very handy when optimizing an existing antenna with

nontelescoping elements and you're unwilling to fabricate new

ones.  When combined with fixed element positions, this option

lets you simply trim existing elements for best performance.

When you disallow a longer boom, YO monitors the

position of the reflector and last director during optimization.

YO allows these elements to migrate closer than their initial

distance, but not farther apart.  YO updates the boom-length

18

limit whenever you reposition or add end elements using the

Elements Menu, you use Undo or Redo, or you disallow a longer

boom.  The boom-length limit also affects the mouse.

Select optimization at the middle frequency to optimize

a design at a spot frequency while keeping an eye on response

elsewhere.

YO provides a MININEC calibration for comparison

purposes.  It's identical to that used in YO 3.0.  It's not as

accurate as the NEC calibration.

You can disable the element-current profile to reduce

screen clutter.

When you select Save Options/Colors, YO creates or

updates the YO.INI initialization file.  YO saves current

settings for all parameters in the Options and Tradeoffs menus

and color codes for all screens in this file.  Whenever YO

begins execution, it checks the current directory for YO.INI.

If it exists, YO restores all parameters from the file.  Use

Save Options/Colors to customize YO's default settings (pressing

S from a Color Menu does the same thing).

YO can generate output files with tapered or with

electrically equivalent untapered elements.  You can use

untapered AO output files for simplified models (they'll use

fewer segments and run faster).  Use tapered AO output files for

highest accuracy.  It's best to generate tapered output files

for NEC/Yagis so it can use its own tapering algorithm.

YO automatically converts dimensions in Yagi files to

the units specified in the Options Menu.  Output files also use

these units.

---- GAIN FIGURE-OF-MERIT --------------------------------------

YO performs an approximate calculation for the maximum

forward gain possible on the current boomlength.  For free-

space, single-Yagi models, YO displays the difference between

the current gain and this maximum at each frequency.  This

number provides a very handy gain figure-of-merit (FOM) that

shows at a glance how close your design is to realizing maximum-

possible gain.

To calculate maximum-possible gain, YO uses a simple

formula derived from a linear-regression curve-fit done by Tom

19

Ring, WA2PHW, on an extensive collection of high-performance

Yagi designs:

G = 10Log(5.4075B + 4.25)

where G is gain in dBd and B is boomlength in wavelengths.

In addition to the WA2PHW gain formula, YO provides two

others.  The first, from Rainer Bertelsmeier, DJ9BV, computes

the gain of DL6WU-design long-boom Yagis.  VHF/UHF Yagis

designed by Guenter Hoch, DL6WU, have become something of a

standard for high-performance, state-of-the-art designs:

G = 7.773Log(B) + 9.28

The last gain formula is from Bill Myers, K1GQ, for HF

Yagis:

G = 3Ln(B) + 9.85

YO uses the WA2PHW formula by default because it gives

good results for any boom longer than 0.25 wavelength.

None of the formulas represent theoretical absolute-

maximum-gain designs.  This condition leads to extremely low

impedances and impractical antennas.  The formulas were derived

from gains of practical, near-maximum-gain Yagis with real-world

losses and design compromises.  Because of this, some

exceptional designs have positive FOMs.  It's easy to change the

coefficients of the gain formula to suit more- or less-stringent

conditions, to better reflect your own experience, or to fit

theoretical calculations.  See the SET Commands section for

details.

The gain FOM is not intended as an overall quality

rating for Yagis.  It rates gain/boomlength efficiency only.

Most Yagi applications require good patterns and this constraint

always places a limit on forward gain.

---- ELEMENT-CURRENT PROFILE -----------------------------------

YO draws a profile of element-current magnitudes along

the Yagi sketch.  Current is plotted linearly, where zero

coincides with the boom.  YO normalizes the profile so that

curve height remains constant from screen to screen.  You can

scale curve height with the up-arrow and down-arrow keys.

Scaling is useful to bring out small detail or to position the

curve for screen printout.  You can scale the profile during

optimization without interrupting the optimizer.  Use the

Options Menu to disable the curve.

The element-current profile is fascinating to ponder for

its deeper significance, but it also has a practical use.  You

can use it to identify elements with questionable tuning,

20

particularly for long-boom Yagis.  Elements with abnormally high

or low current are easily spotted in the profile.

YO computes the element-current profile at the middle

frequency.  The profiles at the low and high frequencies may be

quite different and are interesting to compare.

---- FREQUENCY GRAPHS ------------------------------------------

YO graphs forward gain, F/R, SWR, and impedance curves

versus frequency.  Regular command keys are active while viewing

graphs.  For example, you can press M and G to alternate between

the Match Menu and graphs when adjusting a matching network.  F5

changes screen colors, while PrtSc prints the screen.

YO scales graphs automatically.  It uses 41 frequency

steps.  To change this number, see the SET Commands section.

The F/R curve reflects current settings for F/R region and

average/peak backlobe power.  The impedance curve graphs input

resistance.  YO graphs spot-frequency designs over a +/- 0.5%

frequency range.

You can save the current graph by pressing F8 (the first

graph viewed is saved automatically).  Later, you can recall a

saved graph for comparison by pressing the spacebar.  The

spacebar then switches instantly between the two graphs.  Better

still, you can overlay two graphs by pressing O.  This is a

great way to compare the performance of two designs.

---- PATTERN PLOTS ---------------------------------------------

YO displays patterns using the standard ARRL log-dB

polar scale.  Half-scale is about 12 dB down and quarter-scale

is about 24 dB down.  You can directly compare YO patterns with

those in ARRL publications since both use the same scale.  To

reduce screen clutter, YO draws pattern sketches without grids

or scale markings.  The sketches have 5-degree resolution.

Use the Plot command to generate high-resolution

patterns with grids, scale markings, and annotation.  YO

calculates these patterns at the middle analysis frequency with

1-degree resolution.  The Plot command generates a plot file and

then displays it.  YO saves patterns in files so that you can

review and compare them later.  A plot file uses the Yagi

filename and the extension .PF.  If you plot, change antenna

geometry, and press P again, YO overwrites the plot file with

new patterns.  But if you press Alt-P, YO generates a new plot

file.  Each new file has an incrementing digit appended to its

name.  Use Alt-P to avoid overwriting plot files generated

earlier in a session.  (For both P and Alt-P, YO writes a plot

file only when the pattern has changed.)

You can review a plot from DOS by typing YO PLOT.  This

bypasses the analysis part of YO and lists plot files in the

21

current directory.  Specify a directory name to list plot files

in a different directory.  You can specify the name of a plot

file, for example, YO PLOT PV4, to display a pattern

immediately.

YO writes plot files using the OpenPF plot-file format.

OpenPF is an open, nonproprietary standard for files containing

electromagnetic-field data.  See OPENPF.DOC for details.  YO

creates OpenPF plot files with phase data for free-space

elevation patterns.  These plot files provide complete elevation

data for use with the TA Terrain Analyzer program.  YO centers

the Yagi on the boom when writing a plot file to provide a

consistent phase reference.

YO can use either of two radial scales for polar plots.

The standard ARRL log-dB scale causes minor lobes to be

compressed toward the center of the pattern, emphasizing major

lobe shape.  This scale is widely used in amateur publications

and provides a convenient way to compare the patterns of

antennas you develop with those of existing designs.  On this

scale the center of the plot is minus-infinity dB (no response),

but there is little plot area below -40 dB.

The other polar scale is a linear-dB scale that cuts off

at -50 dB at the center of the plot.  This scale provides much

more area between -20 and -50 dB.  It's useful for examining

minor lobes that may be hard to see in a log-dB plot.

Dots in the sparse radial lines are spaced 2 dB apart.

Dots in the outer two circles are spaced 1 and 2 degrees apart.

These calibrations allow you to read directivity values from

polar plots with good accuracy.

Select rectangular coordinates with the R key.  This

coordinate system can reveal minor-lobe detail even better than

the linear-dB polar plot, but the overall pattern shape isn't as

easy to grasp.  The X-axis represents degrees off boresight and

the Y-axis is antenna response in dB.  You can change the Y-axis

cutoff with the Y key.  This parameter is always negative, but

you can enter it without a minus sign for convenience.  Change

the X-axis cutoff with the X key.  The value you enter is

rounded to the nearest multiple of 10 degrees.

Use the O key to overlay two patterns.  YO displays

filenames instead of plot titles for overlays.  YO coordinates

overlay annotation with patterns by color.  Besides overlays,

you can do switched-plot comparison by pressing C.  After

selecting a second plot file, use the spacebar to switch between

the two.  This is good for comparing patterns with very detailed

sidelobe structure that may get muddled in an overlay.

YO identifies free-space patterns as E-Plane or H-Plane.

It uses the annotations Azimuth or Elevation for over-ground

patterns.

22

YO PLOT normally begins by drawing a polar plot of the

azimuth/E-plane pattern using the log-dB scale.  If you specify

a filename on the command line, you can add up to three option

letters to start in a different mode:

E       Elevation/H-plane pattern

R       Rectangular plot

L       Linear-dB scale

YO plots are perfectly circular on monitors with

standard 4:3 aspect ratio.  Adjust your monitor's vertical-

height control to correct elliptical plots.

---- ARRAYS OF YAGIS -------------------------------------------

YO can synthesize the pattern and gain for an m by n

rectangular array of Yagis given the pattern of a single

antenna.  Since m and n can have any value and array synthesis

is very fast, you can quickly investigate the properties of very

large Yagi arrays.

Press P to plot a single-Yagi pattern.  Then press the +

key to increment the antenna count in the plane you're viewing.

Press the - key to decrement the count.  Press Enter to switch

to the other plane and use + and - to set the Yagi count in that

plane.

Vary array spacing with the up/down arrow keys.  Use the

Home and End keys for finer resolution and PgUp and PgDn for

coarser.  Array spacing varies in the plane you're viewing.

Spacing can differ in the two planes, but all Yagis are spaced

uniformly within a plane.  YO uses wavelength spacing units by

default.  Press the U key to change to inches or millimeters.

You can compare the pattern of a synthesized array with

another pattern, but you can't normalize patterns or overlay

them.  YO synthesizes free-space patterns only.

YO ignores mutual impedances between individual Yagis

when synthesizing array patterns.  Interaction is small for most

Yagis at the spacings typically used for VHF/UHF stacked arrays.

YO uses pattern integration to estimate array gain.  Estimated

gain is within a few tenths of a dB of true array gain.  More

important, it peaks at a spacing close to that where true gain

peaks.  By varying array spacing and observing the gain figure

and pattern sidelobes, you can quickly determine the spacing

that yields a desired gain/pattern trade-off.  Then analyze the

whole array with NEC/Yagis to account for all mutual impedances.

---- ALGORITHM LIMITATIONS -------------------------------------

YO closely predicts actual antenna performance as long

as you accurately characterize element tapering and mounting and

you follow the guidelines in this section.

23

YO is calibrated to NEC, the Numerical Electromagnetics

Code.  MININEC-based antenna-analysis programs may give

different results.  The MININEC algorithm has an inherent

frequency-offset error.  MININEC gives results similar to those

of NEC but at a frequency up to 2% higher, depending on element

diameter.  Use the Options Menu to compare NEC and MININEC

calibrations.

YO closely tracks NEC for element diameters up to 0.01

wavelength (0.27" at 432 MHz).  Yagis with thicker elements may

exhibit some frequency offset from calculated designs, although

YO gives reasonable agreement with NEC for diameters up to about

0.04 wavelength.

YO uses a narrowband modeling algorithm.  Accuracy may

degrade at frequencies more than about 5% away from the central

design frequency.

YO models with input impedances of a few ohms are not

likely to be accurate.  At these impedance levels element

currents are very large and fields nearly cancel.  This

condition greatly magnifies small model inaccuracies.  Low-

impedance Yagis should be avoided for practical reasons as well.

Dimensions become critical, skin effect can cause considerable

loss, impedance matching becomes difficult, and bandwidth is

severely restricted.

Input-impedance accuracy decreases for maximum-gain or

long-boom designs.  YO typically is a couple ohms high for

maximum-gain designs with impedances under 10 ohms.  It can be a

few ohms low for designs with a dozen or more elements and

impedances in the 20- to 30-ohm range.

Modeling accuracy may degrade for elements spaced closer

than about 0.05 wavelength.  In practice, HF elements spaced

this close may move enough in the wind to affect the response of

critical designs.

Accuracy may decrease somewhat for Yagis with element

half-lengths shorter than about 0.19 wavelength.  Elements this

short normally aren't used except for very long-boom designs.

Use NEC/Yagis to verify these designs.

To prevent gross modeling errors, YO won't let you enter

element half-lengths shorter than 0.15 wavelength or longer than

0.3 wavelength.  It won't let you position elements closer than

0.05 wavelength.  Finally, it won't let you enter taper

diameters greater than 0.05 wavelength.

---- CHANGING SCREEN COLORS ------------------------------------

Press F5 to change colors for any screen.  Use the left-

and right-arrow keys to select a screen item (an item blinks

once when selected).  Then use the other keys listed to adjust

the red, green, and blue color components.  When you're done

24

changing colors, press Esc to run YO with temporary new colors.

Press S instead of Esc to make your color changes permanent.  S

saves color codes in the YO.INI initialization file.  YO saves

colors for all screens whenever it saves those for one.

YO.INI also contains codes for many menu settings.

Whenever YO finds this file in the current directory on startup,

it initializes menus and colors to the saved values.  To

customize YO to your way of working, set the menus and colors

according to your preferences and then select Save Options/

Colors in the Options Menu (or press S from the Color Menu).

To reset colors and menus to the defaults, simply delete

YO.INI.

---- PRINTING THE SCREEN ---------------------------------------

YO prints screens to HP LaserJet-compatible printers

(including DeskJets) and to Epson-compatible, dot-matrix

printers.  Press PrtSc to print any screen.  See the SET

Commands section to configure YO for your particular printer.

YO prints monochrome images only.

To keep the mouse cursor from appearing in a screen

print, move it off the right edge of the screen.  To erase the

tradeoffs triangle for printout, click the mouse while off

screen.

Unless you're in LaserJet landscape mode, YO does not

eject a page after printing a single screen.  This lets you

print two screens on one page (24-pin prints are too big to

allow this).  YO will eject the page automatically after it

prints a second screen.  To eject a single-page print, press

F12.

---- GRAPHICS IMAGE FILES --------------------------------------

YO can save the image of any screen in the .PCX file

format.  This feature lets you add YO graphics to desktop-

publishing and word-processing documents.  If your computer has

a fax/modem and your fax software reads .PCX files, you can fax

YO images.  YO generates monochrome .PCX images only.

Press F9 to generate a .PCX file.  The output filename

is the Yagi filename with the extension .PCX.  Whenever you

press F9 again, YO appends an incrementing digit to the filename

and creates another file.  You can control image size and

centering with a DOS SET command.  See the SET Commands section

for details.

25

---- MOUSE -----------------------------------------------------

YO can make use of a mouse.  Although you can use it to

select Yagi files and Main Menu items, its main purpose is to

enable you to change antenna geometry and tradeoffs graphically.

Grab an element at its center to change its position.

Grab it at the top to change its length.  If you have a steady

hand, you can grab an element at the bottom to vary position and

length simultaneously.  To undo mouse changes, click on Undo.

You can even adjust elements during optimization.  This is handy

for nudging the optimizer toward a geometry you believe to be

optimum.

If YO detects a mouse, it displays a triangle whenever

the Main Menu disappears.  The vertices of the triangle

represent tradeoffs.  Forward gain is on the left, F/R is on the

right, and SWR or impedance is on top.  The dot within the

triangle represents the current tradeoff settings.  Grab the dot

with your mouse and move it to alter the tradeoffs.  When you

release the mouse button, the optimizer starts automatically.

Grab the dot during optimization to change tradeoffs

dynamically.

The real purpose of the tradeoffs triangle is to give

you a simple, intuitive way to explore Yagi performance space on

very fast computers.  For example, small Yagis converge almost

instantly on Pentium processors.  Simply move the dot around

within the triangle to see which designs seem most attractive.

On a very fast machine the design displayed is always optimal.

Pressing the right mouse button has the same effect as

pressing the Esc key.

---- SET COMMANDS ----------------------------------------------

DOS provides a convenient way to specify configuration

information to YO.  The DOS SET command places information you

supply into the DOS environment in memory where it can be

retrieved later by a program.  It's convenient to put most SET

commands in your AUTOEXEC.BAT file so they'll be executed

automatically every time the computer boots.  You can enter SET

commands in upper or lower case.  It's best not to leave spaces

around the = in SET commands.

26

You can see what's in the DOS environment by typing the

following:

SET

You can eliminate a SET parameter P by typing this:

SET P=

If you use lots of SET commands, it's possible to run

out of environment space.  To enlarge it to 1024 bytes, put a

line similar to this one in your CONFIG.SYS file:

SHELL=\COMMAND.COM /E:1024 /P

1. Subdirectories

Once you accumulate many Yagi and plot files, it's nice

to organize them into subdirectories.  You might use the current

directory for experiments, saving optimized Yagi files and plots

elsewhere.  You can tell YO to automatically reference certain

subdirectories with SET commands.  YO uses the subdirectories

only for reading files; it always writes OUT.YAG, plot files,

and .PCX files to the current directory.

Define subdirectories like this:

SET YAG=WIREBEAM VHF KLM .

SET PF=. HFPLOTS VHFPLOTS WIREBEAM

The dot represents the current directory.  It can appear

anywhere (or nowhere) in a list.  You can define as many

subdirectories as you like.  You may find it convenient to use

the same set of subdirectories for both Yagi and plot files.

You can organize subdirectories by Yagi type, frequency band,

designer, etc.

Without SET commands, the subdirectory lists consist of

just the current directory.

YO (YO PLOT) begins by listing files in the first

subdirectory on the YAG (PF) list.  After the files it lists the

names of the other subdirectories.  Select one to list its

files.

You can specify a file on the command line from one of

the subdirectories without typing its path.  YO searches the

subdirectories in the order they appear in the subdirectory

list.

You can specify a subdirectory (not necessarily from the

list) on the command line to list its files.  To list files in

the current directory when it isn't first on the list, type YO .

or YO PLOT ..

27

2. Gain Reference

To display gain figures in dBd rather than dBi, do the

following:

SET DB=dBd

To use dBi over ground and dBd in free space, try this:

SET DB=dBdi

Gain in dBd is referenced to the peak gain of a halfwave

dipole in free space.  Gain in dBi is referenced to an isotropic

antenna in free space.  An isotropic antenna radiates equally in

all directions.  A dipole has 2.15 dB gain over an isotropic

antenna, so YO converts from one gain reference to the other by

adding or subtracting 2.15 dB.

3. Mouse Sensitivity

To reduce mouse sensitivity by a factor of two, do this:

SET MOUSESENS=HALF

For a factor of four, try this:

SET MOUSESENS=QUARTER

4. Plot Normalization

YO normally scales the low-, middle-, and high-frequency

patterns independently.  This gives each plot the same amplitude

in the forward direction, making it easy to compare patterns.

However, gain differences aren't apparent.  To show these, you

can normalize plots so that all patterns are drawn to the same

scale.  To set this up, do the following:

SET NORM=ON

5.  Display Typeface

You can use the standard serif typeface built into your

VGA card with the following:

SET TYPEFACE=Serif

6. Printer Type

YO defaults printed output for HP LaserJet or DeskJet

printers in portrait mode.  To try a landscape plot (not

available on all printers), use:

28

SET PINS=HPLJ L

For a bigger landscape plot, use:

SET PINS=HPLJ LB

YO normally draws LaserJet/DeskJet plots with a border.

To eliminate the border, add an X to other options, for example:

SET PINS=HPLJ LBX

YO can transfer compressed data to PCL 5 laser printers

for faster output.  To try this, add a C to other options, for

example:

SET PINS=HPLJ LBXC

For a 9-pin dot-matrix printer, do the following:

SET PINS=9

Some 9-pin printers do not recognize the special line

spacing command YO uses to print an exact screen image.  If your

printer won't print YO screens, try the following:

SET PINS=9ALT

Even with 9ALT, YO may be unable to print to certain old

9-pin printers.

For a 24-pin dot-matrix printer, use:

SET PINS=24

7. Printer Port

Output goes to LPT1 by default.  For LPT2 or LPT3, use

one of the following:

SET LPT=LPT2

SET LPT=LPT3

8.  .PCX Output

By default, YO generates .PCX screen images with 640 by

480 pixels.  You can incorporate these images in documents

directly.  However, the images won't be centered on fax pages

because they have no margins.  You can add margins with the

following command:

SET PCX=left top bottom

The three numbers specify margin size in inches.  The

bottom margin is optional.  You can use it to pad the image to

29

form a complete page if your fax software doesn't do this

automatically.  Try left = .8 and top = 2.35 for 8.5" x 11"

pages.

You can create a double-size .PCX image by adding the

keyword BIG to the line (upper or lower case).  Use BIG for

high-resolution fax mode.

9. Notepad Editor Keys

You can redefine the keys the notepad editor uses for

word delete and line delete with WDEL and LDEL.  Use ^ to

indicate the Ctrl key and Alt- to indicate the Alt key in

combination with a letter key.  You can also use ^Enter and

^Bksp.  Use F1 through F10 for function keys.  For example:

SET LDEL=^Y

SET WDEL=Alt-K

SET LDEL=F3

SET WDEL=^Bksp

10.  Typematic Control

When you press and hold a key, the key repeats after a

short delay.  Default PC values are 500 ms delay and a repeat

rate of 10 characters per second (10.9 cps for PS/2).  Many

users prefer a shorter delay and a faster repeat rate.  YO sets

the delay to 250 ms and the repeat rate to 24 cps.  You can

change these values as follows:

SET TYPEMATIC=delay rate exitdelay exitrate

Delay codes are 0-3 as follows:

0    250 ms     2   750 ms

1    500        3  1000

Rate codes are 0-13 as follows:

0   30.0 cps    7   16.0 cps

1   26.7        8   15.0

2   24.0        9   13.3

3   21.8       10   12.0

4   20.0       11   10.9

5   18.5       12   10.0

6   17.1       13    9.2

The exitdelay and exitrate parameters are optional.

When present, YO sets these typematic values on exit to DOS.

When not present, the values on exit are those in effect while

running YO.

30

To keep YO from modifying your system's typematic

values, do this:

SET TYPEMATIC=0

11. Number of Frequency Steps for Graphs

YO uses 41 frequency steps for graphs by default.  If

you have a slow computer, you may want to generate graphs

faster.  If you have a fast computer, you might like higher

graph resolution.  You can change the number of frequency steps

with the following:

SET NFREQ=Number

12. Minimum Element Spacing

YO won't let you position elements closer than 0.05

wavelength.  It maintains the same limit during automatic

optimization.  To change the spacing limit, do the following:

SET MINSP=Minimum Spacing

13. Linear-dB Plots

YO normally displays pattern sketches and high-

resolution plots using a log-dB polar scale.  You can specify a

linear-dB scale with:

SET POLAR=LINEAR

The linear-dB scale cuts off at -50 dB at the polar

center.

14. Gain Figure-of-Merit

To change the coefficients used in the WA2PHW gain

formula, do the following:

SET FOM=WA2PHW a b

where G = 10Log(aB + b).  G is gain in dBd and B is boomlength

in wavelengths.  If a and b are not specified, default values

31

are used.  Default values for the WA2PHW formula are a = 5.4075

and b = 4.25.

To select the DL6WU gain formula, use:

SET FOM=DL6WU a b

where G = aLog(B) + b.  Default coefficients are a = 7.773 and b

= 9.28.

To use the K1GQ gain formula, enter:

SET FOM=K1GQ a b

where G = aLn(B) + b.  Default values are a = 3 and b = 9.85.

When experimenting with the coefficients, note that a

controls the rate at which gain increases with boomlength, while

b affects the gain at all boomlengths.

When there is no FOM SET command, YO uses the WA2PHW

gain formula with default coefficients.  You can disable the FOM

display entirely by setting FOM to anything other than WA2PHW,

DL6WU, or K1GQ.  For example:

SET FOM=OFF

32

INDEX

#12 wire  15

* key  5

+ key  23

- key  23

.ANT  18

.PCX  25, 29

.YAG  1

180 to 180 degrees  3

4:1 balun  16

5% of the element half-length  7

6061-T6  9

6063-T832  9

= key  5, 7

^  30

~ key  5

Above 30 MHz  15

Absolute-maximum-gain designs  20

Accuracy  1, 19, 24

Algorithm  6

Algorithm Limitations  23

Allow a longer boom  18

Alt key  30

Alt-D  18

Alt-P  21

Alt-R  13

Alt-U  13

Alt-W  18

Alt-Z  13

Aluminum  9

Always make the matching network adjustable!  16

Analysis plane  3

AO  17

AO file format  18

AO output files  19

AO-format output file  17

Arrays of Yagis  23

ARRL  22

Arrow key  1, 2, 5, 18, 23

AUTOEXEC.BAT  26

Automatic optimization of phasing-line impedance  16, 17

Average line impedance  17

Azimuth  22

Backlobe power  3, 21

Backlobes  3

Backspace  18

Balun  14, 15

Band edges  6

Bandwidth  9

Beep  13

Bertelsmeier  20

Best set of antenna dimensions  12

33

Bias  6

BIG  30

Bigger landscape plot  29

Blink  5, 24

Boom Compensation  8

Boom-correction formula  8

Boom-length limit  18

Boomlength  19, 31

Border  29

Bracket Menu  7

Brass  9

C  10

Change taper diameters while maintaining the same electrical design  10

Clamps  8, 15

Coefficients  20

Color  21, 24

Color codes  19

Command key  13

Command line  1, 2, 27

COMMAND.COM  27

Compare patterns  28

Complex self-impedance  10

Compressed data  29

Conductivity  4

Conductivity Losses  9

CONFIG.SYS  27

Conjugate-gradient optimizer  12

Constant spacing between driven elements  16

Constructing a Yagi  16

Converge  10

Copper  9

Count elements  5

Crossover spacing  17

Ctrl  18, 30

Current  20

Current directory  1, 27

Current profile  20

Current source  17

Current Yagi design  13

Cursor  5, 18

Cylindrical equivalents  6

D key  5

DBd  28

DBi  28

Default  2, 3, 11, 12, 16, 19, 20, 23, 25, 29, 31, 32

Del  4, 18

Delete a word  18

Design sensitivity  5

DeskJet  25, 28

Dielectric constant  4

Dimension tolerances  5

Dimensions in Yagi files  19

Dipole  28

Directory  1, 22, 27

34

Disable the good-enough thresholds  12

Disables automatic adjustment  14

Display Typeface  28

DJ9BV  20

DL6WU  8, 20, 32

DOS SET command  26

Dot  27

Dot-matrix printer  29

Dot-matrix printers  25

Down-arrow  20

Driven element  14

Driven-element diameter  16

Dual driven elements  5, 16

Dummy reflector  5

E-plane  18, 22

Earth noise  3

Effective diameter  16

Eject  25

Element currents  3

Element position  18

Element reactances  10

Element self-impedances  6

Element Tapering  5

Element-current distribution  9

Element-current profile  19, 20

Element-mounting brackets  7

Elements  31

Elements Menu  4, 16

Elements with questionable tuning  20

Elevation  22

Elevation angle  2, 4

Elliptical plots  23

EME arrays  3

End  1, 18, 23

Enter  1, 2, 13, 17

Epson-compatible  25

Equivalent diameter  6

Esc  2, 26

Estimate array gain  23

Expand or shrink the design  5

Experiment  2, 5, 11, 17, 27, 32

F/B  3

F/R  3, 10

F/R region  3, 21

F/R tradeoff  10

F12  25

F5  21, 24

F8  21

F9  25

Fast computer  26, 31

Faster output  29

Favor a certain portion of a band  14

Fax  25, 29

Feed impedance  16

35

Feedline  14

Ferromagnetic material  9

Figure-of-merit  19

Final dimensions  5

First design  13

Fixed element dimensions  5

Folded dipole  15

FOM  2, 19, 32

Forward gain  1, 10

Free space  4

Frequency band  3, 6, 9, 16

Frequency bias  6

Frequency Graphs  21

Frequency Menu  3

Frequency range  21

Frequency Scaling  9

Frequency steps  21, 31

Frequency-offset error  24

Front-to-back  3

Function keys  30

G3SEK  8

Gain  23

Gain differences  28

Gain figure-of-merit  2, 19, 31

Gain FOM  20

Gain formula  31

Gain Reference  28

Gain tradeoff  10

Gain/boomlength efficiency  20

Gain/pattern trade-off  23

Gamma Match  15

Generalized F/B  3

Global optimum  12

Good-Enough Thresholds  11

Gradient  12

Graph resolution  31

Graphics Image Files  25

Ground  4

H-plane  3, 18, 22

Hairpin Match  14

Halfwave dipole  28

Heavily tapered elements  6

Height  8

Height Menu  4

HF  15, 16, 20

HF Yagis  3

Hide the Main Menu  2

High-resolution patterns  21

High-resolution plots  31

Highlight  5

Highlighted file  1

History of design changes  13

Hoch  8, 20

36

Home  1, 18, 23

Hy-Gain element clamps  15

Idealized matching network  14

Impedance  2, 4, 14, 15, 21, 24

Impedance tradeoff  11

Impractical antennas  20

Inches  23

Input resistance  11

Input-impedance modeling error  16

Ins  4

Insulated through-the-boom mounting  8

Intermediate designs  13

Interrupt the optimizer  13

Isotropic antenna  28

Iteration count  3

Jaggard  7

Joint position  16

K1GQ  20, 32

Key repeats  30

KLM  16, 17

L-network  14

L4 norm  3, 11

Lambda symbol  2

Landscape plot  28

LaserJet  25, 28

Lawson  6

LDEL  30

Lead length  15

Leeson  6, 7

Length of a single driven element  5

Lightbar  1

Limit  11

Limitations  13

Line delete  18, 30

Linear-dB  31

Linear-regression curve-fit  19

Listing files  27

Local optimum  13

Log-dB scale  23

Logarithmic average  16

Long-boom designs  24

Long-boom Yagis  20

Low impedances  20

LPT  18, 29

Main Menu  2

Match frequency  14

Match Menu  5, 14, 16

Matching Networks  14

Maximum forward gain  19

Maximum-possible gain  19

Menu settings  25

37

Middle frequency  19

Migrate closer  11, 18

Millimeters  23

Minimum Element Spacing  31

MININEC  1, 6, 24

MININEC calibration  19

Modified-W2PV algorithm  6

Mounting bracket  7

Mouse  1, 19, 26

Mouse cursor  25

Mouse sensitivity  28

Multiple peaks  13

Mutual impedance  1, 4, 23

Myers  20

NEC  1, 24

NEC/Yagis  4, 19, 23, 24

Network  14

Nontelescoping elements  18

Normalize  23

Normalize plots  28

Notepad  18

Notepad Editor Keys  30

Notes  18

Number of Frequency Steps for Graphs  31

Numerical Electromagnetics Code  1, 24

Ohmic losses  9

Omnidirectional noise  10

OpenPF plot-file format  22

Optimal dimensions  12

Optimization Method  12

Optimization summing weights  10

Optimize joint driven-element position  17

Optimum SWR characteristics  17

Option letters  23

Options Menu  3, 4, 6, 18

Other  2

OUT.YAG  17, 27

Output filename  18

Output Files  17, 19

Overlay  22, 23

Parameter tradeoffs  10

Parameters  19

Path  27

Pattern integration  23

Pattern Plots  21

PCL 5 laser printers  29

PCX  25, 29

Peak backlobe  3

Peak power  3

Pentium processors  26

Percent  10

Perfect match  14

Permeability  9

38

PF  21

PgDn  1, 18, 23

PgUp  1, 18, 23

Phase data  22

Phasing line  16

Phasing-line tension  16

Phosphor bronze  9

Physical Design of Yagi Antennas  6

Plane  23

Plot file  2, 21, 27

Plot Normalization  28

Polar scale  21

Polarization  4

Power  3

Power within the F/R region  3

Power-computation method  3

Print an output file  18

Printer Port  29

Printer Type  28

Printing The Screen  25

PRN  18

Profile  20

Proximity of other antennas  16

PrtSc  18, 21, 25

Quick check  2

Radial scales  22

Rear horizon  3

Recall a previous design  13

Recall a saved graph  21

Recalled design  14

Rectangular array  23

Redo  13, 19

Region  3

Relative importance  10

Replicate a Yagi design  9

Resistance  11

Resistivity  9

Resolution  23

Resonant length  14

Respaces elements  5

Return to the Main Menu  2

Right mouse button  26

Ring  19

RMS power  3

Rods  14

Rounding  5

Running YO  1

Save command  13

Save Options/Colors  19

Save the current graph  21

SAVE.YAG  17

Scale  9, 28

Screen Colors  24

39

Self-impedance  6, 10

Self-impedance errors  6

Serif typeface  28

SET DB  28

SET FOM  31

SET LDEL  30

SET LPT  29

SET MINSP  31

SET MOUSESENS  28

SET NFREQ  31

SET NORM  28

SET PF  27

SET PINS  29

SET POLAR  31

SET TYPEFACE  28

SET TYPEMATIC  30

SET WDEL  30

SET YAG  27

SHELL  27

Shortened driven element  14

Shunt capacitance  15

Shunt inductance  14

Shunt-capacitance estimation error  16

Sidelobe structure  22

Sidelobes  3

Signal-to-noise ratio  10

Silver  9

Sketches  31

Skin effect  24

Skin-effect  9

Slow computer  31

Source magnitudes and phases  17

Spacebar  2, 13, 21

Spacers  17

Spacing between phasing straps  17

Spacing limit  31

Speed  4, 14

Spot frequency  3, 9, 19

Spot-frequency  21

Stack  13

Stacking distance  4

Stainless steel  9

Strategy for optimizing a dual-driven Yagi  17

Subdirectories  27

Subdirectory list  27

Submenu  2

Switched-plot comparison  22

SWR  10, 11, 17

SWR bandwidth  10

SWR reference impedance  14

SWR tradeoff  11

Synthesize  23

T Match  15

TA Terrain Analyzer  22

Tab key  7, 14

40

Taper conversion  6

Taper schedule  6

Taper Schedule Menu  7

Tapering algorithm  5

Terminate data entry  2

Terminating optimization  13

Through-the-boom mounting  8

Tip length  10

Tolles  15

Tradeoff percentages  11

Tradeoffs  10

Tradeoffs Menu  3, 10, 16

Tradeoffs triangle  26

Transmission line  14

Triangle  25, 26

Trim existing elements for best performance  18

True array gain  23

Tweezers  4

Two-element Yagi with driven element and director  5

Typematic Control  30

U key  23

Undo  13, 19

Units  18, 19

Untapered elements  19

Untapered-equivalent lengths  5

Up-arrow  20

Upper or lower case  26

Velocity factor  16

W2PV  1, 6, 8, 10

W6QHS  6, 7

W7ITB  15

WA2PHW  19, 31

Wavelength  23

WDEL  30

Weighted combination  12

Weighted sum  10

White  8

Wild-goose chases  12

Word delete  18, 30

Wraps words  18

X key  5

X-axis cutoff  22

Y-axis cutoff  22

Yagi Antenna Design  6

Yagi files  1

Yagi performance space  26

Yagi sketch  5, 20

Yagi title  18

YO analysis kernel  6

YO PLOT  2, 21

YO-format output files  18

YO.INI  19, 25

Z  2

41